# Northern Digital Inc. Aurora 
Electromagnetic Tracking Equipment:

EQUIPMENT PRODUCES MAGNETIC FIELDS

CAN BE HARMFUL FOR PACEMAKERS AND OTHER SENSITIVE EQUIPMENT

Usage Notes by Andrew Razjigaev 2023

Introduction to NDI Aurora

NDI Aurora System:
More details on NDI Aurora and latest products can be viewed online:

https://www.ndigital.com/electromagnetic-tracking-technology/

# Setup in the Lab:

Note, ensure the cable to the NDI aurora electromagnetic field generator is not wound up:

Installation for the first time: file:///C:/Program%20Files%20(x86)/Northern%20Digital%20Inc/ToolBox/readme.htm#_USB_Drivers_Installation

- From the Force Dimension, USB, put the “Northern Digital Inc” folder into 
Program Files (x86)

- In addition, copy the folder “CombinedAPISample” to any location like the desktop.

- Connect the USB port for NDI aurora

- In the start menu, find “control panel” -> “Device Manager”

- In device manager, find under “other” the “NDI Aurora SCU Port”

- With Admin privileges, change settings and update the driver. Manually browse computer for drivers in : Program Files (x86)\Northern Digital Inc\ToolBox\USB Driver

- Repeat 5-6 until a port allocated e.g. COM3

- In the “Northern Digital Inc” folder run Track.exe and connect to COM3 (or whatever port was connected)

# Checking The Tracking

Run Track.exe and the GUI should automatically show the workspace and the sensor’s location whether it is trackable or outside the workspace and the electromagnetic field generator:

Note all activity with the tracker are saved in a logfile called capi.txt

# Usage for gathering data using CombinedAPISample:

- In project folder CombinedAPISample, go to Source -> Debug

- Run the executable: NDI CombinedAPI Sample.exe (Alternatively if editing the source code in Visual studio, open the CombinedAPISample.sln and run the program there)

- In the executable, Initialize system, Activate Handles and Start Tracking to record data

- In the same folder (Source -> Debug), a file called NDIdata.csv would be recording data in the form of:

- The data records the time, position (X,Y,Z) and quaternion (qo,qx,qy,qz) for each of the 4 sensors (a,b,c,d). Note all activity with the tracker are also saved in a logfile called capi.txt

- You can process the data in MATLAB. My repository has the code: https://github.com/Andrew-Raz-ACRV/ndi_tracker_project Use the command NDI_csr_to_vectors and the data in the .csv file becomes a structure of data that can be used.

# Modifying the source code of CombinedAPISample:

- The source code can be modified in visual studio using the desktop development tools in C++. It may not compile successfully at first due to the settings in visual studio. To make it compile modify the visual studio installer setup to have MSVC tools:

- In project folder CombinedAPISample, go to Source and find the following file: CombinedAPISampleDlg.cpp which handles the communication of data. By searching ‘Andrew’ one can find my modifications for creating the .csv file capability. This modified piece of code can be found in GitHub: https://github.com/Andrew-Raz-ACRV/ndi_tracker_project